ECEN 5623						REAL-TIME EMBEDDED SYSTEMS					SATYA MEHTA

Homework set 4																	Date: 04/16/2019

- Develop an example of a 32-bit Hamming encoded word (39 bits total) and show a correctable SBE scenario.  Show the data word in a table like Figure 5.6 in the book.

Check bits is not equal to 0 and pw2 != pw, Hence there is SBE, we can correct. By check bits we found that there is a flip on 23rd bit from 0 to 1, Hence we corrected it to 0.

- For the foregoing problem, now show an uncorrectable MBE scenario.

There is MBE as we can see that Check bits !=0 and pw2=pw, Hence it is uncorrectable.

- For the following Nand flash block update history for 2 sectors that contain 4 blocks each (e.g. 16K sectors, with 4K blocks), fill in the missing WRITE operations as needed and compute write-amplification.

#1 – All blocks FREE

#2 – Erase S0 & S1, WRITE: - LB 0,1,2,2

#3 – Read LB 0, 2, Modify, WRITE: - LB 0,2

#4 – Read LB 1, 3, Modify, WRITE: - LB 1,3

#5 – Read LB 0, 2, Modify and Cache

#6 – Buffer LB 0, 1, 2, Erase S0

#7 – WRITE- LB 0,1,2 to S0

Write Amplification = 11/10 = 1.1

#8 - Read LB 1, 3, Modify and Cache

#9 – Erase S1

#10 – WRITE- LB 1,3 to S1

#11 – Read LB 0, 2, Modify, WRITE LB 0,2

#12 – Read LB 1, 3, Modify and Cache

#13 – Erase S0

#14 – WRITE – LB 1,3

Write Amplification = 17/16 = 1.0625

Total sector erases for both S0 and S1 = 5 Erases